# Launches and First-Time Run

Every launch file with its exact command, where to put your API keys, single-view vs multi-view grasping, the three ways to run GraspGen, and how to run repeated-pick experiments.

# 0. Every session: source the workspace

source /opt/ros/humble/setup.bash
source ~/new_ur_gripper_ws/install/setup.bash

After editing any code, rebuild:

cd ~/new_ur_gripper_ws && colcon build --symlink-install && source install/setup.bash

# 1. API keys — where to insert them

LLM credentials live in ~/new_ur_gripper_ws/.morty_env (chmod 600). Edit it and fill in your keys:

nano ~/new_ur_gripper_ws/.morty_env

| Variable | Purpose |
| --- | --- |
| OPENAI_API_KEY_DIRECT | OpenAI API key (sk-...). |
| OPENAI_MODEL | OpenAI model (e.g. gpt-5.4-mini). |
| AWS_BEARER_TOKEN_BEDROCK | AWS Bedrock bearer token (ABSK...). |
| AWS_REGION | Bedrock region (e.g. us-east-1). |
| BEDROCK_MODEL | Bedrock model id (e.g. anthropic.claude-haiku-4-5-...). |

Select a provider (morty.sh does this automatically):

source ~/new_ur_gripper_ws/scripts/use_openai.sh     # OpenAI direct
source ~/new_ur_gripper_ws/scripts/use_bedrock.sh    # AWS Bedrock

Note: Source a provider (or run morty.sh) BEFORE launching the chat node. Keep .morty_env private; rotate shared keys.

# 2. All launch files — exact commands

## 2.1 communication_llm (voice / LLM)

### morty_full.launch.py

The whole system: voice + perception + MoveIt executor. (morty.sh runs this.)

ros2 launch communication_llm morty_full.launch.py use_fake_hardware:=true

### voice_robot.launch.py

Communication side only: TTS, STT, chat, intent router, gripper + named-pose bridges.

ros2 launch communication_llm voice_robot.launch.py

## 2.2 grasp_pipeline (perception + grasping)

### run_grasp_pipeline.launch.py

YOLO detection + segmentation + point cloud + GraspGen client.

ros2 launch grasp_pipeline run_grasp_pipeline.launch.py

### open3d_cloud_viewer.launch.py

Standalone Open3D point-cloud viewer (debug).

ros2 launch grasp_pipeline open3d_cloud_viewer.launch.py

## 2.3 ur3e_gripper_3d_moveit_config (robot + MoveIt)

### ur3e_gripper_3d_moveit.launch.py

UR3e + gripper + MoveIt + ros2_control. Simulation by default.

ros2 launch ur3e_gripper_3d_moveit_config ur3e_gripper_3d_moveit.launch.py use_fake_hardware:=true

### ur3e_gripper_3d_moveit_real.launch.py

Same, but connects to the physical UR3e.

ros2 launch ur3e_gripper_3d_moveit_config ur3e_gripper_3d_moveit_real.launch.py

### ur3e_gripper_3d_moveit_real_camera.launch.py

Real robot AND RealSense camera driver.

ros2 launch ur3e_gripper_3d_moveit_config ur3e_gripper_3d_moveit_real_camera.launch.py

### moveit_rviz.launch.py

RViz with the MoveIt motion-planning panel (usually included by the above).

ros2 launch ur3e_gripper_3d_moveit_config moveit_rviz.launch.py

### setup_assistant.launch.py

MoveIt Setup Assistant (config editing).

ros2 launch ur3e_gripper_3d_moveit_config setup_assistant.launch.py

## 2.4 ur3e_gripper_3d_moveit_commander (arm executors)

### grasp_executor_lin_llm.launch.py

LIN pick with interactive drop pause (used by morty_full).

ros2 launch ur3e_gripper_3d_moveit_commander grasp_executor_lin_llm.launch.py use_fake_hardware:=true

### grasp_executor_lin.launch.py

LIN pick, no drop pause.

ros2 launch ur3e_gripper_3d_moveit_commander grasp_executor_lin.launch.py use_fake_hardware:=true

### grasp_executor.launch.py

Default PTP pick executor.

ros2 launch ur3e_gripper_3d_moveit_commander grasp_executor.launch.py use_fake_hardware:=true

### grasp_executor_pilz.launch.py

Pick using the Pilz industrial motion planner.

ros2 launch ur3e_gripper_3d_moveit_commander grasp_executor_pilz.launch.py use_fake_hardware:=true

### observation_ptp_commander.launch.py

Move the arm through observation/scan poses (PTP).

ros2 launch ur3e_gripper_3d_moveit_commander observation_ptp_commander.launch.py use_fake_hardware:=true

## 2.5 ur3e_fast_observation (named-pose teach/replay)

### teach.launch.py

Record observation poses (joint snapshots).

ros2 launch ur3e_fast_observation teach.launch.py

### replay.launch.py

Replay taught poses (fast, bypasses MoveIt).

ros2 launch ur3e_fast_observation replay.launch.py

## 2.6 gripper_3d (gripper + camera hardware)

### ur3e_gripper_3d.launch.py

UR3e + gripper URDF/description display.

ros2 launch gripper_3d ur3e_gripper_3d.launch.py

### run_gripper_u3re_camera.launch.py

UR3e + gripper display, optional real RealSense (use_real_camera:=true).

ros2 launch gripper_3d run_gripper_u3re_camera.launch.py

### realsense_d345_hardware.launch.py

RealSense D345/D435 camera driver only.

ros2 launch gripper_3d realsense_d345_hardware.launch.py

### gripper_io_hardware.launch.py

Gripper I/O control hardware bringup.

ros2 launch gripper_3d gripper_io_hardware.launch.py

# 3. Single-view vs multi-view grasping

Plan a grasp from one camera view (single-view) or move the arm to several observation poses and TSDF-fuse them (multi-view). Controlled by registration_enabled.

| Mode | Meaning |
| --- | --- |
| registration_enabled:=false | Single-view: one capture, faster, no arm sweep. |
| registration_enabled:=true | Multi-view: arm sweeps observation poses, TSDF-fuses, denser cloud. |

Defaults differ by launcher:

• morty_full.launch.py starts SINGLE-view (registration_enabled:=false).

• run_grasp_pipeline.launch.py (standalone) starts MULTI-view (registration_enabled:=true).

Set it at launch:

# multi-view with Morty:
ros2 launch communication_llm morty_full.launch.py use_fake_hardware:=true registration_enabled:=true

# single-view with the standalone pipeline:
ros2 launch grasp_pipeline run_grasp_pipeline.launch.py registration_enabled:=false

Or flip it live, without restarting:

ros2 param set /yolo_live_select_grasp registration_enabled true     # enable multi-view
ros2 param set /yolo_live_select_grasp registration_enabled false    # back to single-view

Note: With Morty, voice command 'enable multi-view' / 'disable multi-view' does the same.

# 4. Running GraspGen — three options

## Option A — In-process (no server)  [recommended]

GraspGen runs inside the pipeline; no service, no ZMQ port. Default mode.

export GRASPGEN_MODE=inprocess        # default; can be omitted
export GRASPGEN_SAVE_VIZ=~/grasp_viz  # optional: auto-render each grasp to PNG
ros2 launch grasp_pipeline run_grasp_pipeline.launch.py

Note: First grasp ~10 s (model load), then ~1 s. GRASPGEN_SAVE_VIZ writes latest.png with cloud + grasps via tools/grasp_debug_viewer.py.

## Option B — Standalone server (ZMQ)

Run GraspGen as a separate service. Useful to keep the model loaded across pipeline restarts, or to run it on a different machine.

# terminal A — service (+ MeshCat viewer):
cd ~/new_ur_gripper_ws && ./scripts/start_graspgen.sh
#   without viewer:   ./scripts/start_graspgen.sh --no-viz

# terminal B — pipeline in zmq mode:
export GRASPGEN_MODE=zmq
ros2 launch grasp_pipeline run_grasp_pipeline.launch.py graspgen_endpoint:=tcp://127.0.0.1:5555

Note: MeshCat viewer: http://127.0.0.1:7000/static/ . For a remote machine, bind the service with --bind tcp://0.0.0.0:5555 and point graspgen_endpoint at its IP.

## Option C — Server via Docker (only if needed)

Containerized GraspGen (matches the lab PC). Needs Docker + the NVIDIA Container Toolkit + a built graspgen image; heavier than A or B.

cd ~/new_ur_gripper_ws/GraspGen/docker && ./build.sh    # one-time image build
./run_graspgen_service.sh                               # run the service container
# then run the pipeline in zmq mode (Option B, terminal B).

Note: Use Docker only to mirror the lab machine. Options A and B need no Docker.

# 5. Running multi-view grasping experiments

tools/run_grasp_experiment.sh wraps run_grasp_pipeline.launch.py in experiment mode: it forces multi-view + dynamic NBV + TSDF, records the full log + a summary log + structured JSON/CSV, and survives Ctrl-C (the summary/CSV/JSON are written even if you interrupt).

## 5.1 One experiment run

# default: 9-class YOLO model, multi-view + dynamic NBV + TSDF
bash ~/new_ur_gripper_ws/tools/run_grasp_experiment.sh --tag vitamin_tube_run1

# use the older 3-class model
bash ~/new_ur_gripper_ws/tools/run_grasp_experiment.sh --tag retail_box_old1 --model-preset old

# pass extra ros2 launch overrides at the end
bash ~/new_ur_gripper_ws/tools/run_grasp_experiment.sh --tag run42 registration_motion_timeout_s:=150

What gets written under $WS_DIR/src/experiment_logs/ for each run:

| File | Contents |
| --- | --- |
| <ts>_<tag>.log | Full raw stdout/stderr of the run. |
| <ts>_<tag>_summary.log | Greps the key NBV / sweep / grasp-utility lines. |
| <ts>_<tag>_meta.txt | Run metadata (model, classes, launch args). |
| <ts>_<tag>_structured.json | Parsed runs/views (via parse_grasp_experiment.py). |
| <ts>_<tag>_run.csv | One row per run. |
| <ts>_<tag>_views.csv | One row per accepted view. |

## 5.2 Aggregate, plot, report many runs

# 1. (auto-done by the runner) parse a single raw log:
python3 ~/new_ur_gripper_ws/tools/parse_grasp_experiment.py \
        --raw-log <ts>_<tag>.log \
        --json-out <ts>_<tag>_structured.json \
        --run-csv-out <ts>_<tag>_run.csv \
        --views-csv-out <ts>_<tag>_views.csv

# 2. merge all runs in experiment_logs/ into one aggregate CSV:
python3 ~/new_ur_gripper_ws/tools/aggregate_grasp_experiments.py

# 3. matplotlib plots from the aggregate:
python3 ~/new_ur_gripper_ws/tools/generate_experiment_plots.py

# 4. HTML report from the aggregate:
python3 ~/new_ur_gripper_ws/tools/build_experiment_report.py

Note: All four python scripts use argparse; run with --help for the full flag list.

## 5.3 Other tools (in tools/)

• grasp_debug_viewer.py — render cloud + grasps to a PNG (also invoked by GRASPGEN_SAVE_VIZ).

• capture_yolo_dataset.py — capture camera frames + manifest for YOLO fine-tuning.

• export_ros_graph.py — snapshot the live ROS node/topic graph (DOT/SVG/PNG) for thesis figures.