# Fast Run — Cheat Sheet

The short version: how to start the system day-to-day, with and without the voice/communication layer, on the real robot or in simulation.

# 1. Full system with voice (Morty)

morty.sh is the one-command launcher. It sources ROS + workspace + an LLM provider, then brings up the application layer (voice + perception + executor). Bring up the robot in a separate terminal first.

# terminal 1 — the robot + MoveIt:
#   simulation:
ros2 launch ur3e_gripper_3d_moveit_config ur3e_gripper_3d_moveit.launch.py use_fake_hardware:=true
#   real robot:
ros2 launch ur3e_gripper_3d_moveit_config ur3e_gripper_3d_moveit_real.launch.py

# terminal 2 — Morty (GraspGen runs in-process by default):
cd ~/new_ur_gripper_ws
./morty.sh --fake          # OpenAI + simulated robot
./morty.sh                 # OpenAI + REAL robot
./morty.sh --debug --fake  # cheap Bedrock LLM + simulated robot
./morty.sh --voice-only    # chat + gripper + named poses, no grasping

| morty.sh flag | Effect |
| --- | --- |
| (none) | OpenAI gpt model + real hardware |
| --fake | Use simulated (mock) robot — no real UR3e/camera |
| --debug | Use AWS Bedrock (cheaper dev LLM) instead of OpenAI |
| --voice-only | Skip perception + executor (chat + gripper + named poses only) |
| --fast-voice | Faster, lower-quality TTS/STT (pyttsx3 + whisper-tiny) |
| --help | Show all options |

Note: Match the LLM/hardware: if terminal 1 is fake, use ./morty.sh --fake so the executor expects mock hardware too.

# 2. Grasp pipeline only (no voice) + simple execution

To run perception + grasping + arm motion WITHOUT the chat/voice layer — e.g. to test picking by clicking the object in the debug window or via a topic.

# terminal 1 — robot + MoveIt (sim or real, as above)
ros2 launch ur3e_gripper_3d_moveit_config ur3e_gripper_3d_moveit.launch.py use_fake_hardware:=true

# terminal 2 — perception + GraspGen (in-process, no server needed)
ros2 launch grasp_pipeline run_grasp_pipeline.launch.py

# terminal 3 — the arm executor that performs the pick
ros2 launch ur3e_gripper_3d_moveit_commander grasp_executor_lin.launch.py use_fake_hardware:=true

Select an object without voice — by index on a topic:

ros2 topic pub --once /yolo/select_index std_msgs/Int32 "{data: 0}"

Note: Object selection without voice: click it in the debug window, point at it (MediaPipe), or publish an index on /yolo/select_index. The grasp pipeline then builds the cloud, calls GraspGen, and the executor moves the arm to the chosen grasp.

# 3. Real system vs. fake system — the one rule

| Layer | Simulation | Real hardware |
| --- | --- | --- |
| Robot/MoveIt | ur3e_gripper_3d_moveit.launch.py use_fake_hardware:=true | ur3e_gripper_3d_moveit_real.launch.py |
| Executor | ...launch.py use_fake_hardware:=true | ...launch.py use_fake_hardware:=false |
| Morty | ./morty.sh --fake | ./morty.sh |
| Camera | not available (no live detections) | realsense_d345_hardware.launch.py or _real_camera |

# 4. Handy commands

# source everything (each new terminal)
source /opt/ros/humble/setup.bash
source ~/new_ur_gripper_ws/install/setup.bash

# rebuild after code changes
cd ~/new_ur_gripper_ws && colcon build --symlink-install && source install/setup.bash

# health checks
ros2 node list
ros2 action list                    # /execute_grasp, /move_action, /speak, /gripper
ros2 control list_controllers       # arm + gripper should be 'active'
ros2 topic echo /joint_states --once